48V / 3kW PMSM

Motor Controller Board

Complete Parameter Reference for PCB Design

Design Requirements Summary

| System Target | Value | Notes |
| --- | --- | --- |
| Bus Voltage (nominal) | 48 V | Li-ion / LiFePO4 pack |
| Bus Voltage (peak) | 60 V | Regen / overshoot maximum |
| Output Power | 3,000 W | 3 kW continuous |
| Max Phase Current | 80 A | Peak, before OCP trip |
| Switching Frequency | 20,000 Hz | 20 kHz, above audible |
| Topology | 3-Phase PMSM | Field-Oriented Control (FOC) |
| MOSFET Count | 6× | High + Low per phase arm |
| Gate Driver Count | 3× | One per half-bridge |

TOTAL INPUTS: 58 parameters driving 12 calculation modules producing ~120 design outputs

SECTION A — System-Level Specifications (Manual Entry: Settings Tab)

These are your design targets — entered once before touching any datasheet. Every calculation module references at least one of these values.

All 7 parameters are mandatory. Default values are pre-filled but must be verified against your actual system.

| Parameter | Symbol | Default | Unit | Why It Matters |
| --- | --- | --- | --- | --- |
| Bus Voltage (nominal) | V_bus | 48 | V | Used in every conduction/switching loss formula and capacitor sizing |
| Bus Voltage (peak) | V_peak | 60 | V | Snubber voltage rating, OVP threshold, MOSFET Vds derating margin |
| Output Power | P | 3000 | W | DC bus current (P/V_bus), capacitor sizing, total thermal budget |
| Max Phase Current | I_max | 80 | A | Gate resistors, shunt values, OCP threshold, PCB trace widths |
| PWM Switching Freq | f_sw | 20000 | Hz | All switching losses, ripple current, dead time, EMI filter corner |
| Ambient Temperature | T_amb | 30 | °C | Thermal derating — Tj = T_amb + P_loss × Rth_total |
| Gate Drive Voltage | V_drv | 12 | V | Gate resistor calc, bootstrap capacitor, driver VCC rail design |

SECTION B — MOSFET Datasheet Extractions (6× devices, extract from one)

You use 6 identical MOSFETs — one high-side + one low-side per phase arm. Extract all parameters below from a single MOSFET datasheet. Conditions matter: extract at VGS=10V (or your gate drive voltage) and TC=25°C unless noted.

CRITICAL: Always extract values at the correct test conditions. Rds(on) at VGS=4.5V is useless if you drive at 10V. Check footnotes carefully.

## B1. Electrical Parameters — Critical for Loss Calculations

| Parameter | App ID | Unit | Condition | Drives Calculation |
| --- | --- | --- | --- | --- |
| Max Drain-Source Voltage | vds_max | V | Absolute max | Safety margin check: V_peak < 0.7 × Vds_max |
| Continuous Drain Current | id_cont | A | TC=25°C | Sanity check: must be ≥ I_max |
| On-Resistance | rds_on | mΩ | VGS=10V,25°C | Conduction loss = I²_rms × Rds(on) × 1.5 (3-phase factor) |
| Gate Threshold Voltage | vgs_th | V | ID=1mA | Bootstrap minimum voltage check, driver UVLO validation |
| Total Gate Charge | qg | nC | VGS=10V | Bootstrap cap size, driver peak current, gate resistor power |
| Gate-Drain Charge (Miller) | qgd | nC | VGS=10V | DOMINANT switching loss term; dV/dt = I_drive / Cgd |
| Gate-Source Charge | qgs | nC | VGS=10V | Turn-on delay portion of switching loss |
| Output Capacitance | coss | pF | VDS=V_bus | Snubber capacitor: Cs = 3 × Coss snapped to E12 series |
| Reverse Recovery Charge | qrr | nC | IF=I_max | Recovery loss = Qrr × V_bus × f_sw per FET |
| Reverse Recovery Time | trr | ns | IF=I_max | Dead time minimum constraint: dt > trr |
| Turn-On Delay | td_on | ns | Rg external | Used in switching loss integral, dead time timing |
| Rise Time | tr | ns | Rg external | Switching loss = ½×V_bus×I_max×(tr+tf)×f_sw; dV/dt |
| Turn-Off Delay | td_off | ns | Rg external | DEAD TIME = td_off + tf + 20% margin (most critical) |
| Fall Time | tf | ns | Rg external | Switching loss; dead time minimum |
| Body Diode Forward Voltage | body_diode_vf | V | IF=I_max | Freewheeling conduction loss during dead time window |

## B2. Thermal Parameters — Critical for Heatsink Design

| Parameter | App ID | Unit | Notes | Drives Calculation |
| --- | --- | --- | --- | --- |
| Junction-to-Case Thermal Resistance | rth_jc | °C/W | Case = device bottom / tab | Tj = T_case + P_loss × Rth_jc; heatsink design |
| Maximum Junction Temperature | tj_max | °C | Absolute max rating | Thermal margin = Tj_max − Tj_estimated; safety flag |

## B3. How Each MOSFET Parameter Feeds the Calculations

| Parameter | Calculation Module | Output |
| --- | --- | --- |
| rds_on | calc_mosfet_losses() | P_cond = I²_rms × Rds(on) × 1.5 |
| qg | calc_bootstrap_cap() | C_boot = Qg / ΔV_droop × 2 (safety margin) |
| qg | calc_gate_resistors() | Driver peak current = Qg / t_rise |
| qgd (Miller) | calc_gate_resistors() | dV/dt = I_source / Cgd; switching loss |
| tr, tf | calc_mosfet_losses() | P_sw = ½ × V_bus × I_max × (tr+tf) × f_sw |
| td_off, tf | calc_dead_time() | dt_min = td_off + tf + 20% margin |
| coss | calc_snubber() | Cs = 3 × Coss → snapped to E12 standard |
| qrr | calc_mosfet_losses() | P_recovery = Qrr × V_bus × f_sw |
| trr | calc_dead_time() | dead time cross-check: dt > trr |
| rth_jc, tj_max | calc_thermal() | Tj_est, thermal margin, copper pour area |

SECTION C — Gate Driver Datasheet Extractions (3× ICs, one per half-bridge)

One driver IC drives one half-bridge (high-side + low-side MOSFET pair). You use 3 identical driver ICs for 3-phase. Extract all parameters from a single driver datasheet.

## C1. Power Supply / Bias — Critical

| Parameter | App ID | Unit | Drives |
| --- | --- | --- | --- |
| VCC Supply Range | vcc_range | V | Power supply design, VCC bypass capacitor sizing, startup sequencing |
| VCC UVLO Threshold | vcc_uvlo | V | Minimum supply voltage for operation; fault/startup logic |
| Max Bootstrap Voltage | vbs_max | V | Bootstrap capacitor voltage rating selection |
| Bootstrap UVLO | vbs_uvlo | V | Minimum bootstrap voltage; cross-check vs C_boot design |

## C2. Gate Drive Capability — Critical for Rg Minimum

| Parameter | App ID | Unit | Drives |
| --- | --- | --- | --- |
| Peak Source Current (turn-on) | io_source | A | Rg_on minimum = (V_drv − V_gs_miller) / I_source |
| Peak Sink Current (turn-off) | io_sink | A | Rg_off minimum = V_gs_miller / I_sink |

Rg must be large enough to limit di/dt and dV/dt, but cannot be smaller than the driver current limit. The calculated Rg from switching time requirements is then clamped to this minimum.

## C3. Timing Parameters — Critical for Dead Time

| Parameter | App ID | Unit | Drives |
| --- | --- | --- | --- |
| Propagation Delay Turn-On | prop_delay_on | ns | Actual dead time the MCU must program (asymmetric comp) |
| Propagation Delay Turn-Off | prop_delay_off | ns | Dead time asymmetry: Δprop = prop_on − prop_off |
| Min Dead Time (internal) | deadtime_min | ns | Cross-check: dt_recommended must be ≥ deadtime_min |
| Default Dead Time | deadtime_default | ns | Starting point for firmware tuning |

## C4. Logic Interface — For MCU GPIO Compatibility

| Parameter | App ID | Unit | Drives |
| --- | --- | --- | --- |
| Input Low Voltage | vil | V | MCU GPIO output VOL must be below this for logic 0 |
| Input High Voltage | vih | V | MCU GPIO output VOH must be above this for logic 1 |

## C5. Protection Parameters — For System Safety Design

| Parameter | App ID | Unit | Drives |
| --- | --- | --- | --- |
| OCP Threshold | ocp_threshold | A or mV | Hardware overcurrent protection trip point (shoot-through guard) |
| OCP Response Time | ocp_response | ns/µs | Time to shutdown for shoot-through; latency budget |
| Thermal Shutdown Temp | thermal_shutdown | °C | Driver self-protection limit; system thermal budget |
| Thermal Resistance J-A | rth_ja | °C/W | Driver junction temp estimate at full gate current load |

## C6. Optional but Useful

| Parameter | App ID | Unit | Drives |
| --- | --- | --- | --- |
| Current Sense Gain | current_sense_gain | V/V | Shunt resistor: Rshunt = V_adc_target / (I_max × gain) |

SECTION D — MCU Datasheet Extractions (1× device)

The MCU runs FOC firmware and generates the 6 complementary PWM signals. Extractions confirm it is capable of the required control bandwidth and peripheral count.

## D1. Processing — Confirms FOC Viability

| Parameter | App ID | Unit | Drives |
| --- | --- | --- | --- |
| Max CPU Frequency | cpu_freq_max | MHz | FOC loop must complete Clarke/Park/SVPWM in < ½ PWM period (25µs at 20kHz) |
| Flash Size | flash_size | KB | Firmware feasibility — motor control libs need ~64–128KB minimum |
| RAM Size | ram_size | KB | Stack + motor control state buffers; minimum ~20KB for FOC |

## D2. ADC — Critical for Current Sensing Accuracy

| Parameter | App ID | Unit | Drives |
| --- | --- | --- | --- |
| ADC Resolution | adc_resolution | bits | Current sensing accuracy; SNR = bits_used = log₂(V_adc / LSB_mV) |
| Number of ADC Channels | adc_channels | count | Must have ≥ phase current channels + bus voltage + NTC channels |
| ADC Sample Rate | adc_sample_rate | MSPS | Sampling window constraint for single-shunt current reconstruction |

## D3. PWM — Critical for Switching Control

| Parameter | App ID | Unit | Drives |
| --- | --- | --- | --- |
| PWM Timer Count | pwm_timers | count | Must have advanced timer with complementary outputs (e.g. TIM1 on STM32) |
| PWM Resolution | pwm_resolution | bits | Determines minimum duty cycle step and dead-time granularity |
| Dead-Time Resolution | pwm_deadtime_res | ns | dt_actual = ceil(dt_recommended / resolution) × resolution |
| Max Programmable Dead Time | pwm_deadtime_max | ns | Safety check: dt_recommended must be < dt_max |
| Complementary Output Pairs | complementary_outputs | count | Must be ≥ 3 pairs for 3-phase (high+low per arm, with interlock) |

## D4. Peripherals & Power Supply

| Parameter | App ID | Unit | Drives |
| --- | --- | --- | --- |
| SPI Count | spi_count | count | Encoder / resolver / magnetic angle sensor interface planning |
| UART Count | uart_count | count | Debug tuning interface, CAN-FD bridge, telemetry |
| VDD Supply Range | vdd_range | V | MCU power supply rail design |
| Run Current | idd_run | mA | MCU VDD power supply bypass capacitor sizing |
| Operating Temp Range | temp_range | °C | Confirms MCU survives ambient + self-heating in enclosure |
| GPIO Count | gpio_count | count | Fault inputs, enable lines, LED, encoder — confirm enough pins |

SECTION E — Motor Specifications (Manual Entry: Motor Tab)

Motor parameters come from the motor manufacturer datasheet or direct winding measurement (Rph, Lph). These connect motor physics to the electronics design — especially bus capacitor sizing.

Lph (phase inductance) is the most impactful motor parameter. Without it, the bus capacitor ripple current calculation falls back to a worst-case D=0.5 estimate. With it, the accurate formula ΔI = V_bus × 0.25 / (Lph × f_sw) is used.

| Parameter | App ID | Unit | Source | Why It Matters |
| --- | --- | --- | --- | --- |
| Motor Type | type | — | Datasheet | PMSM/BLDC affects control algorithm selection (FOC vs 6-step) |
| Max Speed | max_speed_rpm | RPM | Datasheet | Back-EMF check: Ke × (max_rpm/1000) must be < V_bus |
| Pole Pairs | pole_pairs | — | Datasheet | Electrical freq = mechanical × pole pairs; commutation rate |
| Phase Resistance | rph_mohm | mΩ | Measure / DS | Copper loss = 1.5 × I²_max × Rph; L/R time constant |
| Phase Inductance | lph_uh | µH | Measure / DS | Bus cap ripple current (accurate method); current ripple ΔI |
| Torque Constant | kt_nm_per_a | Nm/A | Datasheet | Torque-current scaling for FOC firmware tuning |
| Rated Torque | rated_torque_nm | Nm | Datasheet | Operating point validation; thermal derating reference |
| Back-EMF Constant | back_emf_v_per_krpm | V/kRPM | Datasheet | Safety: Ke × (max_rpm/1000) < V_bus (must never exceed bus) |

SECTION F — What Gets Calculated (12 Modules, ~120 Outputs)

This is the full data flow: which input parameters feed each calculation module, and what outputs are produced. Use this to verify the logic end-to-end.

| # | Module | Key Inputs | Key Outputs |
| --- | --- | --- | --- |
| 1 | MOSFET Losses | rds_on, tr, tf, qrr, qgd, I_max, V_bus, f_sw | P_cond/FET, P_sw/FET, P_recovery/FET, P_total ×6 FETs, efficiency % |
| 2 | Gate Resistors | qg, qgd, io_source, io_sink, V_drv, tr, tf, f_sw | Rg_on (E24), Rg_off (E24), rise/fall time, dV/dt, Rg power rating |
| 3 | Bus Capacitors | lph_uh (motor), I_max, V_bus, f_sw, P | I_ripple_rms, C_bulk required, N caps, ESR budget, film+MLCC values |
| 4 | Bootstrap Cap | qg (MOSFET), vbs_uvlo (driver), ΔV_droop = 0.5V | C_boot exact, E12 standard value (2× margin), min HS on-time, hold time |
| 5 | Shunt Resistors | I_max, csa_gain (driver), ADC Vref = 3.3V | Rshunt (single + 3-phase), V_shunt@Imax, V_ADC, ADC bits used, power |
| 6 | RC Snubber | coss (MOSFET), I_max, V_peak, stray L (~10nH assumed) | Rs (E24, critically damped), Cs (E12 = 3×Coss), V_overshoot, f_resonant |
| 7 | Protection Dividers | V_bus, V_peak, I_max | OVP R1/R2, UVP R1/R2, OCP hardware threshold, NTC thermistor divider |
| 8 | Power Supply Bypass | V_drv, idd_run (MCU) | VCC bypass per driver IC, MCU VDD bypass, PVCC bulk cap, 5V rail |
| 9 | EMI Filter | V_bus, f_sw, I_max | CM choke inductance, Y-cap value, X-cap value, filter corner frequency |
| 10 | Thermal Analysis | P_total, rth_jc (MOSFET), T_amb, tj_max | Tj estimate, thermal margin °C, heatsink requirement, copper pour area mm² |
| 11 | Dead Time | td_off, tf, trr (MOSFET), prop_delay (driver), dt_res (MCU) | dt_min, dt_recommended, dt_actual (MCU register), dt as % of period |
| 12 | PCB Guidelines | (engineering rules, I_max, f_sw) | Power trace widths (mm), gate trace widths, via sizes, copper pour specs |

SECTION G — Pre-Calculation Completeness Checklist

Use this before running calculations. Every unchecked box is a gap that will cause a fallback estimate or wrong output. Print and check off as you extract from each datasheet.

G1 — MOSFET Datasheet (17 parameters)

| ☐ | vds_max — Max Drain-Source Voltage (V) |
| --- | --- |
| ☐ | id_cont — Continuous Drain Current (A) |
| ☐ | rds_on — On-Resistance (mΩ) at VGS=10V, TC=25°C |
| ☐ | vgs_th — Gate Threshold Voltage (V) at ID=1mA |
| ☐ | qg — Total Gate Charge (nC) at VGS=10V |
| ☐ | qgd — Gate-Drain (Miller) Charge (nC) |
| ☐ | qgs — Gate-Source Charge (nC) |
| ☐ | coss — Output Capacitance (pF) at VDS=V_bus |
| ☐ | qrr — Reverse Recovery Charge (nC) at IF=I_max |
| ☐ | trr — Reverse Recovery Time (ns) |
| ☐ | td_on — Turn-On Delay (ns) |
| ☐ | tr — Rise Time (ns) |
| ☐ | td_off — Turn-Off Delay (ns) |
| ☐ | tf — Fall Time (ns) |
| ☐ | body_diode_vf — Body Diode Forward Voltage (V) at IF=I_max |
| ☐ | rth_jc — Junction-to-Case Thermal Resistance (°C/W) |
| ☐ | tj_max — Maximum Junction Temperature (°C) |

G2 — Gate Driver Datasheet (14 parameters)

| ☐ | vcc_range — VCC Supply Range (V) |
| --- | --- |
| ☐ | vcc_uvlo — VCC Under-Voltage Lockout Threshold (V) |
| ☐ | vbs_max — Maximum Bootstrap Voltage (V) |
| ☐ | vbs_uvlo — Bootstrap Under-Voltage Lockout (V) |
| ☐ | io_source — Peak Gate Source Current (A) |
| ☐ | io_sink — Peak Gate Sink Current (A) |
| ☐ | prop_delay_on — Propagation Delay Turn-On (ns) |
| ☐ | prop_delay_off — Propagation Delay Turn-Off (ns) |
| ☐ | deadtime_min — Minimum Dead Time if built-in (ns) |
| ☐ | deadtime_default — Default Dead Time setting (ns) |
| ☐ | vil — Input Low Voltage threshold (V) |
| ☐ | vih — Input High Voltage threshold (V) |
| ☐ | ocp_threshold — OCP Threshold (A or mV) |
| ☐ | ocp_response — OCP Response Time (ns/µs) |
| ☐ | thermal_shutdown — Thermal Shutdown Temperature (°C) |
| ☐ | rth_ja — Junction-to-Ambient Thermal Resistance (°C/W) |
| ☐ | current_sense_gain— CSA Gain (V/V) if integrated in driver |

G3 — MCU Datasheet (14 parameters)

| ☐ | cpu_freq_max — Maximum CPU Frequency (MHz) |
| --- | --- |
| ☐ | flash_size — Flash Memory Size (KB) |
| ☐ | ram_size — RAM Size (KB) |
| ☐ | adc_resolution — ADC Resolution (bits) — typically 12 |
| ☐ | adc_channels — Number of ADC Channels available |
| ☐ | adc_sample_rate — ADC Sample Rate (MSPS) |
| ☐ | pwm_timers — Number of Advanced PWM Timers |
| ☐ | pwm_resolution — PWM Timer Resolution (bits) |
| ☐ | pwm_deadtime_res — Dead-Time Register Resolution (ns) |
| ☐ | pwm_deadtime_max — Maximum Programmable Dead Time (ns) |
| ☐ | complementary_outputs— Number of Complementary PWM Pairs (must be ≥ 3) |
| ☐ | spi_count — Number of SPI Interfaces |
| ☐ | uart_count — Number of UART Interfaces |
| ☐ | vdd_range — MCU VDD Supply Range (V) |
| ☐ | idd_run — Run Current at max frequency (mA) |
| ☐ | temp_range — Operating Temperature Range (°C) |
| ☐ | gpio_count — Total GPIO Count |

G4 — Motor Datasheet / Measurement (8 parameters)

| ☐ | type — Motor Type (PMSM / BLDC / ACIM) |
| --- | --- |
| ☐ | max_speed_rpm — Maximum Speed (RPM) |
| ☐ | pole_pairs — Number of Pole Pairs (not poles) |
| ☐ | rph_mohm — Phase Resistance (mΩ) — measure with 4-wire Kelvin |
| ☐ | lph_uh — Phase Inductance (µH) — Ld/Lq avg for PMSM |
| ☐ | kt_nm_per_a — Torque Constant Kt (Nm/A) |
| ☐ | rated_torque_nm — Rated Torque (Nm) |
| ☐ | back_emf_v_per_krpm — Back-EMF Constant Ke (V/kRPM) — specify peak or RMS |

G5 — System Specs (7 parameters — Settings Tab)

| ☐ | bus_voltage — Nominal Bus Voltage (V) |
| --- | --- |
| ☐ | peak_voltage — Peak Bus Voltage (V) — regen / worst-case transient |
| ☐ | power — Output Power (W) |
| ☐ | max_phase_current — Maximum Phase Current (A) |
| ☐ | pwm_freq_hz — PWM Switching Frequency (Hz) |
| ☐ | ambient_temp_c — Ambient Temperature (°C) |
| ☐ | gate_drive_voltage— Gate Drive Voltage (V) |

SECTION H — Key Formula Reference

The most important engineering formulas used by the calculation engine. Verify these manually if any result looks unexpected.

| # | Formula | Parameters | Notes |
| --- | --- | --- | --- |
| 1 | P_cond = I²_rms × Rds_on × 1.5 | I_max, rds_on | 1.5 = √1.5² factor for 3-phase RMS (I_rms = I_peak/√2) |
| 2 | P_sw = ½ × V_bus × I_max × (tr+tf) × f_sw | V_bus, I_max, tr, tf, f_sw | Per FET switching loss; multiply ×6 for full bridge |
| 3 | P_recovery = Qrr × V_bus × f_sw | qrr, V_bus, f_sw | Body diode reverse recovery, per FET |
| 4 | dt_min = td_off + tf + 20% margin | td_off, tf | Minimum dead time to prevent shoot-through |
| 5 | C_boot = Qg / ΔV_droop × 2 | qg, ΔV=0.5V | 2× safety margin; snap to E12 cap series |
| 6 | Cs = 3 × Coss → snap to E12 | coss | Snubber cap: 3× MOSFET output capacitance |
| 7 | Rs_crit = √(L_stray / Coss) | stray L (~10nH), coss | Critical damping resistance; clamp 1–100Ω, snap E24 |
| 8a | ΔI = V_bus × D(1−D) / (Lph × f_sw), D=0.5 | V_bus, lph_uh, f_sw | ACCURATE: phase current ripple using motor inductance |
| 8b | I_ripple_rms = I_dc × √0.25 [fallback] | P/V_bus | FALLBACK: generic D=0.5 estimate when Lph not entered |
| 9 | C_req = I_ripple_rms / (8 × f_sw × ΔV) | I_ripple_rms, f_sw, ΔV=2V | Bulk cap required; snap up to N×100µF standard caps |
| 10 | Rg_on = (V_drv − V_gs_miller) / (2 × io_source) | V_drv, io_source, qgd | Gate-on resistor; check dV/dt = io_source/Cgd |
| 11 | Tj = T_amb + P_loss × Rth_jc | T_amb, P_loss, rth_jc | Junction temp; margin = tj_max − Tj |
| 12 | V_BEMF = Ke × (N_max / 1000) | back_emf_v_per_krpm, max_speed_rpm | Must be < V_bus (check on Motor tab) |

SECTION I — Parameters Collected But Not Yet Used in Calculations

These parameters are extracted or entered but currently not wired into a calculation module. They are planned for future implementation.

| Parameter | Source | Current Status | What It Would Improve |
| --- | --- | --- | --- |
| qgs (gate-source charge) | MOSFET | Extracted, not used | More precise turn-on delay in switching loss integral |
| Rated Torque (Nm) | Motor | Entered, not used | Operating point derating; thermal headroom at partial load |
| Max Speed (RPM) | Motor | Back-EMF check only | FOC bandwidth validation: f_bandwidth < f_sw / 10 |
| ADC Sample Rate (MSPS) | MCU | Extracted, not used | Single-shunt reconstruction window feasibility check |
| Flash / RAM Size | MCU | Extracted, not used | Firmware footprint feasibility for motor control library |
| GPIO Count | MCU | Extracted, not used | Pin count sufficiency for full schematic |
| Thermal Shutdown (driver) | Driver | Extracted, not used | Driver self-protection limit in thermal budget |

Summary — Total Input Count

| Source | Parameters | Method |
| --- | --- | --- |
| Section A — System Specifications | 7 | Manual (Settings tab) |
| Section B — MOSFET Datasheet | 17 | AI PDF Extraction |
| Section C — Gate Driver Datasheet | 17 | AI PDF Extraction |
| Section D — MCU Datasheet | 17 | AI PDF Extraction |
| Section E — Motor Specifications | 8 | Manual (Motor tab) |
| TOTAL | 58 inputs | 12 calc modules → ~120 outputs |

Document generated for the 48V/3kW PMSM Motor Controller project. Cross-reference with PROJECT_CONTEXT.md for implementation details.